# WSJT-X User’s Guide

Version 1.0

Joe Taylor, K1JT

Copyright © 2013

May 7, 2013

Contents

Introduction	3

System Requirements	4

Installation and Setup	4

Basic Operating Tutorial	6

Making QSOs	9

On-Screen Controls	10

Menus	15

Keyboard Shortcuts	17

Special Mouse Commands	18

Under the Hood	18

Acknowledgments	20

Appendix A: Installed and Generated Files	20

Appendix B:  The JT9 Protocol	21

Appendix C:  Source Code	22

Appendix D:  Troubleshooting	23

# Introduction

WSJT-X  is a computer program designed to facilitate basic amateur radio communication using very weak signals.  The first four letters in the program name stand for “Weak Signal communication by K1JT”, and the “-X” suffix indicates that WSJT-X started as an experimental branch of program WSJT.

Version 1.0 of WSJT-X offers a new mode called JT9, designed for use on the LF, MF, and HF bands.  JT9 shares many characteristics with the modes JT65 and JT4 made popular in WSJT.  All three modes are designed for making reliable, confirmed QSOs under extreme weak-signal conditions.  They use nearly identical message structure and source encoding.  JT65 was designed for EME (“moonbounce”) on the VHF/UHF bands and has also proved very effective for worldwide QRP communication at HF; JT4 is used mainly on the microwave bands. In contrast, JT9 is optimized for HF and lower frequencies.  JT9 is about 2 dB more sensitive than JT65A while using less than 10% of the bandwidth.  World-wide QSOs are possible with power levels around 1 W and compromise antennas.  Several dozen JT9 signals fit easily into a 1 kHz slice of spectrum.

All WSJT modes use timed sequences of alternating transmission and reception.  JT9 offers five choices for the sequence durations: submodes JT9-1, JT9-2, JT9-5, JT9-10, and JT9-30 use 1, 2, 5, 10, and 30 minutes, respectively.  A minimal QSO with JT9-1 usually takes 4 to 6 minutes:  2 or 3 transmissions by each station, one transmitting in odd minutes and the other even.  The remaining JT9 submodes take proportionally longer, so JT9-1 is the preferred submode under most circumstances.  The submodes with longer transmissions trade reduced throughput for smaller bandwidth and increased sensitivity.  The slowest submode, JT9-30, has total bandwidth 0.4 Hz and operates at signal-to-noise ratios as low as –40 dB measured in the standard 2.5 kHz reference bandwidth.  It requires very stable oscillators in both transmitter and receiver.  JT9-1 is always the recommended submode unless you really need the additional sensitivity of a slower mode.

Plans for future program development call for WSJT-X and WSJT to merge together: WSJT-X will gradually acquire the modes JT65, JT4, FSK441, and ISCAT that are now supported in WSJT.  The entire WSJT-related effort is an open-source project.  If you have programming skills or would like to contribute to the project in other ways, please make your interests known to K1JT and the rest of the development team.  The project’s source-code repository can be found at http://developer.berlios.de/projects/wsjt/.

# System Requirements

- SSB transceiver and antenna

- Computer running Windows XP or later. (WSJT-X also runs under Linux, OS X, and probably FreeBSD and other unix-like operating systems, but we do not yet provide click-to-install packages for them.  See Appendix C.)

- 1.5 GHz or faster CPU and 512 MB of available memory

- Monitor with at least 1024 x 780 resolution (more is better)

- Audio input and output devices supported by your operating system

- Computer-to-radio interface using a serial port to key your PTT line, or CAT control.  You can also use VOX control for T/R switching.

- Audio or equivalent USB connections between transceiver and computer

- A means for synchronizing your computer clock to UTC.  The built-in Windows facility is usually not adequate.  I recommend Meinberg NTP,  see http://www.satsignal.eu/ntp/setup.html for installation instructions.

# Installation and Setup

- WSJT-X can be downloaded from the WSJT Home Page at http://www.physics.princeton.edu/pulsar/K1JT/. Click on the WSJT link at the left margin and then on the appropriate download link for WSJT-X.  Especially during phases of active program development, consult also the project’s ChangeLog at http://physics.princeton.edu/pulsar/K1JT/wsjtx_changelog.txt.

- Under Windows, execute the downloaded file and follow its installation instructions.  Install WSJT-X into its own directory (the suggested default is C:\WSJTX) rather than the conventional C:\Program Files\WSJTX.  All files relating to WSJT-X will be stored in this directory and its subdirectories.  Nothing is written to the Windows registry, so you can completely uninstall WSJT-X simply by removing the installation directory and its contents.

- Start WSJT-X and Select Configuration from its Setup menu.  Enter your callsign and Maidenhead grid locator as shown in the screen shot on the next page, and set the remaining parameters as required for your station.  A simple setup might use PTT method = VOX and PTT Port = None.   Stations already configured for other digital modes will typically use PTT method = DTR or RTS and a serial communication port such as COM1 for PTT Port.

- Many users will have other software controlling their radios, so WSJT-X does not attempt to implement full transceiver control; it simply provides a way to ensure that WSJT-X can know the radio’s dial frequency and control T/R switching. If you want this capability check the box Enable CAT, select your radio type from a drop-down list, and select a port (not the same port selected for PTT control!) and whatever serial communication parameters may be required by your radio.  If using CAT control, most radios will allow you to set PTT method = CAT, and your radio may support two types of PTT assertion via CAT control: one assuming audio input from the Mic connector, the other from a rear-panel Data connector.  The simplest CAT configuration sets Polling interval to 0 (no polling the radio for dial frequency).  WSJT-X is then able to set the radio’s frequency, but the program will be unaware of subsequent changes made with the radio’s panel controls.  With most radios you can set Polling interval to a reasonable small number (say 1 – 3 s) and the program will then follow any frequency changes made at the radio.  Note that you may not be able to simultaneously control your radio from WSJT-X and from another software program.  Some experimentation may be required, and you may need to refer to the documentation for your radio.  Note: it is best to have the radio and any interface equipment turned on and connected before starting WSJT-X, and to exit the program before turning your equipment off.

- Try clicking the Test CAT Control and Test PTT buttons to see that you have established the desired control of station functions.  Then click OK to dismiss the Configuration window.

- WSJT-X expects your sound card to do its raw sampling at 48000 Hz.  To ensure that this will be so when running under Windows, open the Sound control panel and select in turn the “Recording” and “Playback” devices you will use for audio input and output.  Click on Properties, then Advanced, and select “16 bit, 48000 Hz (DVD Quality).”  If you are using a sound card that is also the default device for Windows sounds, be sure to turn off all such sounds so that you do not inadvertently transmit them over the air.

- To set the proper level of audio drive from WSJT-X to your radio, click the Tune button on the main screen.  WSJT-X should set the radio into transmit mode and generate a steady audio tone at the same amplitude that will be used for a generated JT9 signal.  Listen to the generated audio tone using your radio’s “Monitor” facility, or by another method.  The tone should be perfectly smooth, with no clicks or glitches.  Open the computer’s audio mixer controls for output (“playback”) devices and adjust the “volume” slider downward until the RF output from your transmitter falls by around 10%.  This will be a good level for audio drive.  Toggle the Tune button once more to stop your test transmission.

# Basic Operating Tutorial

- Click the Stop button on the main window to halt any data acquisition.  Select submode JT9-1 from the Mode menu and Normal from the Decode menu.  On the Wide Graph window select JT9 Sync (rather than Current or Cumulative) for data display.  Select Tab 2 (just below the Erase button on the main window) to choose the alternative set of controls for generating and selecting messages to be transmitted.  Then select File | Open, navigate to directory …\Save\Samples under your WSJT-X installation directory, and open the example file 130418_1742.wav.  You should see something like the screen shots on the next page.

- Notice the green, red, and blue markers on the waterfall frequency scale.  Decoding in JT9 mode takes place at the end of a receive sequence and is organized in two stages.  The first decodes take place at the selected Rx frequency, indicated by the green marker.  Results appear in the both the left (“QSO Frequency”) and right (“Band Activity”) text windows on the main screen.  The decoder then finds and decodes all JT9 signals between the blue markers. The normal wideband decoding range is 1000 – 2000 Hz, but you can move the limits using the  f Min and  f Max spinner controls.  The red marker indicates the Tx frequency.

- Note that at least eight JT9 signals are present in the example file; all but one of them are decodable.  When this file was recorded KF4RWA was finishing a QSO with K1JT, and since the green marker was placed at his audio frequency, 1224 Hz, his message “K1JT KF4RWA 73” appears in both decoded text windows.  The “Band Activity” window shows this message as well as all the other decodes at nearby frequencies:  GM7GAX and M0WAY are calling CQ, N7MQ is calling TF3G, N4HFA is calling G7CNF, PD0JAC is sending a signal report to JA1KAU, and N5KDV is calling K1JT as a tail-ender, on a different Tx frequency.  The CQ lines are highlighted in green, and lines containing “My Call”, in this case K1JT, are highlighted in red.  (For this step and the next one, you may want to pretend you are K1JT by entering that call temporarily as “My Call” on the Configuration screen.  Your results should then be identical to those shown here.)

- To gain some feeling for the controls you will use when making QSOs, try clicking with the mouse on the decoded text lines and on the waterfall spectral display.  You should be able to confirm the following behavior:

  - Double-click on either of the decoded lines highlighted in green.  This action copies callsign and locator of a station calling CQ to the “DX Call” and “DX grid” entry fields.  It also generates suitable messages for a minimal QSO and ticks (or clears) the Tx even box so that you will transmit in the proper odd or even minutes.  Rx and Tx frequency markers will be moved to the CQing station’s frequency, and the Gen Msg (“generated message”) radio button at bottom right of the main window will be activated.  If you had selected “Double-click on call sets Tx Enable” on the Setup menu, Enable Tx would also be activated, and you would start to transmit automatically, at the appropriate time.

  - Double-click on the decoded line with the message “K1JT N5KDV EM41”, highlighted in red.  Results will be similar to (a), except the Tx frequency (red marker) is not moved.  Such messages are usually in response to your own CQ, or from a tail-ender.  You probably want your Tx frequency to stay where it was.  By holding down the Ctrl key when double-clicking on the decoded line, you can cause both Tx and Rx frequencies to be moved.

  - Now double-click on the message from KF4RWA in either window.   He is sending “73” to K1JT, signifying that the QSO is over.  Most likely you want to call CQ again, so the message “CQ K1JT FN20” is automatically generated and selected for your next transmission.

  - Clicking on the waterfall moves the Rx frequency (green marker) to the selected frequency.  Ctrl-click on waterfall moves both Rx and Tx frequencies.

  - Double-click on the waterfall moves the Rx frequency and causes a narrow-band decode there at the new QSO frequency.  Decoded text appears in the left window only.

  - Ctrl-double-click moves both Rx and Tx frequencies and decodes at the new frequency.  Again, decoded text appears in the left window.

  - Clicking Erase clears the left window.  Double-clicking Erase clears both text windows.

- Don’t forget to re-enter your own callsign as “My Call”.  Click the Monitor button to return to normal receive operation, and be sure that your transceiver is set to USB (or USB Data) mode.  Using the receiver gain control(s) and/or the Windows mixer controls, set the background noise level to around 30 dB on the thermometer scale at lower left of the WSJT-X main screen.  If necessary you can also use the slider next to the scale, but note that the overall dynamic range will be best with the slider close to mid-scale.  When this is true, the dB scale is calibrated relative to the least significant bit of 16-bit samples from the soundcard.

- You should now be ready to make QSOs with the JT9 modes in WSJT-X.

# Making QSOs

By longstanding tradition, a minimal valid QSO requires the exchange of callsigns, a signal report or some other information, and acknowledgments.  WSJT-X is designed to facilitate making such minimal QSOs using short, formatted messages.  The process works best if you use them and follow standard operating practices.

The recommended basic QSO goes something like this:

1.	CQ K1ABC FN42

2.					K1ABC G0XYZ IO91

3.	G0XYZ K1ABC -19

4.					K1ABC G0XYZ R-22

5.	G0XYZ K1ABC RRR

6.					K1ABC G0XYZ 73

These standard messages consist of two callsigns (or CQ, QRZ, or DE and one callsign) followed by the transmitting station’s grid locator, a signal report, or the acknowledgement “RRR” or sign-off “73”.  Such messages are compressed and encoded in a highly efficient and reliable way.  Signal reports are given as signal-to-noise ratio in dB, using the standard reference noise bandwidth 2500 Hz.  Thus, in example message #2 K1ABC is telling G0XYZ that his signal is 19 dB below the noise power in bandwidth 2500 Hz; in message #3 G0XYZ acknowledges receipt of that report and responds with a –22 dB signal report.  For most operators, signals start to become audible around      –15 dB on this scale.  Signals are visible on the waterfall down to about –26 dB, and the JT9 decoder begins to fail at about this same limit.

Free-format messages such as “TNX JOE 73 GL” or “5W VERT 73 GL” can also be transmitted, up to a maximum of 13 characters.  Users often add some friendly chit-chat as a final transmission, in place of message #6.  It should be obvious, however, that JT9 is not a mode suitable for extensive conversations or “rag-chewing.”

Before attempting your first QSO with JT9, be sure to go through the tutorial above and the following checklist:

- Your callsign and grid locator set to correct values

- PTT and CAT control (if used) properly configured and tested

- Computer clock properly synchronized with UTC to within ±1 s.

- Radio set to USB (upper sideband) mode

- Remember that JT9 generally does not require high power.  QRP is the rule!

# On-Screen Controls

The following row of controls appears at the bottom of the Wide Graph window:

FFT Bins/Pixel gives you control over the displayed frequency resolution.  Set this value to 1 for the highest possible resolution, or to higher values to reduce the necessary width of the spectral display.  Normal operation with convenient window size works well at 2 bins per pixel in submodes JT9-1 and -2.  The slower submodes use proportionally longer FFTs and hence smaller bin widths, so you may want to use larger values.  Waterfall sensitivity to JT9 signals will be best with values no larger than 3 or 4, however.

N Avg is the number of successive FFTs to be averaged before updating the spectral display.  Values around 5 are suitable for normal JT9 operation.

Gain and Zero control amplitude scaling and zero-setting for the waterfall colors.  Values around 0 for both parameters are usually about right, but your preferences may differ somewhat.

f Min and f Max set the positions of the blue markers on the frequency scale — the lower and upper limits of the second (wideband) pass through the JT9 decoder.

The Current/Cumulative/JT9 Sync drop-down list offers three possibilities for graphical display in the bottom one-third of the Wide Graph window.  Current is the average spectrum over the most recent N Avg FFT calculations.  The Cumulative curve is the average spectrum since the start of the current Rx sequence.  Finally, JT9 Sync displays the program’s tentative estimate of whether a JT9 signal is present at each frequency.  (See the discussion under Receiving on page 19 for more details.)

With the obvious exception of f Min and f Max, controls on the Wide Graph window affect only the appearance of what is displayed during a receiving sequence.  They have no effect on the decoding of JT9 signals.

These buttons appears just under the decoded text windows on the main window:

Log QSO pops up a confirmation screen pre-filled with known information about a QSO you have just completed.  You may edit or add to this information before clicking OK to log the QSO.  If you select “Prompt me to log QSO” on the Setup menu (see below), the program will pop up the confirmation screen automatically when you send a “73” or free-text message.

Stop will stop normal data acquisition in case you want to open and explore previously recorded audio files.

Monitor restarts normal program operation in receive mode.  This button is highlighted in green when the program is receiving.

Decode tells the program to repeat the decoding procedure at the QSO frequency (green marker on waterfall), using the most recently acquired sequence of Rx data.  Holding “Shift” down when clicking Decode will initiate decoding at the QSO frequency and then over the full frequency range, f Min to f Max.

Erase clears the left (QSO frequency) window.  Double-clicking Erase clears both text windows.

Tune may be used to switch into Tx mode and generate an unmodulated carrier at the specified Tx frequency (red marker on waterfall).  This process may be useful for adjusting an antenna tuner, for example.  Toggle the button a second time to terminate the Tune process.

Enable Tx puts the program into automatic Rx/Tx sequencing mode and highlights the button in red.  A transmission will start at the beginning of the selected (odd or even) sequence.

Halt Tx will terminate a transmission in progress and disable automatic Rx/Tx sequencing.

At lower left of the main window are controls and displays related to date and time, frequency, Rx audio level, and the station you may be in QSO with.

The drop-down Band selector at upper left lets you select the operating band and sets dial frequency to a default value taken from the Setup | Configuration | Default Frequencies tab.  If you are using CAT control, a small colored square appears in red if the CAT control is two-way between WSJT-X and your radio, or orange if the control is only from program to radio.  If the Dx Grid is known, the great-circle azimuth and distance are given.  The program can keep a database of callsigns and locators, for future reference.  Click Add to insert the present call and locator in the database; click Lookup to retrieve the locator for a previously stored callsign.

At center and right of the main window are a number of controls you will use when making QSOs.  The following are near screen center:

Select Tx even to transmit in even-numbered UTC minutes (or, for the slower JT9 submodes, even-numbered sequences, starting with 0 at the top of a UTC hour).  Uncheck this box to transmit in the odd intervals.  This selection is made automatically when you double-click on a decoded text line (as described in the Basic Operating Tutorial, pages 6 – 9).  Your audio Tx frequency is displayed and can be controlled by the Tx +xxxx Hz spinner control.  Again, this setting is normally handled automatically by the double-click procedure.  The on-the-air frequency of your lowest JT9 tone will be the sum of dial and audio frequencies.  You can force Tx frequency to the current Rx frequency by clicking the Tx=Rx button, and vice-versa for Rx=Tx.  Finally, the Report spin control lets you change a signal report that may have been inserted automatically.  Valid signal reports fall in the range –50 to +49 dB.  For the JT9-1 submode most reports will fall in the range –26 to +10 dB.  When signals are stronger than about 0 dB, you and your QSO partner should probably switch to CW or SSB, or reduce power.  JT9 is supposed to be a weak signal mode!

Two configurations of controls are provided for generating and selecting Tx messages:

Traditional controls carried over from WSJT provide six fields for message entry.  Pre-formatted messages for the standard minimal QSO (see page 9) are generated when you click Generate Std Msgs, or when you double-click on an appropriate line of decoded text.  Select the next message to be transmitted (at the start of your next Tx sequence) by clicking on the circle under “Next”.  To change to a specified Tx message immediately, perhaps after a transmission has already started, click on a rectangular button under the “Now” label.  Changing Tx messages after a transmission has started necessarily reduces the probability of a correct decode by your QSO partner, but if you do it in the first 10 s or so of a Tx period, it will probably succeed.

Right-clicking on the entry field for message #5 pops up a list of free text messages you entered on the Setup | Configuration | Tx Macros dialog window, for example

You can select any of these pre-stored messages with the left mouse button.

The second configuration of message-selecting controls looks like this:

With this setup you will normally follow a top-to-bottom sequence of transmissions from the left column (if you are calling CQ) or the right column (if you are answering a CQ).  Clicking a button puts the appropriate message in the Gen Msg box.  If you are already transmitting, it changes the Tx message immediately.  (The actual message being transmitted always appears in the first box on the status bar, at the bottom left of the main screen.)  You can put anything you like (up to 13 characters) in the Free Msg box. Right-clicking on this entry field pops up your previously defined list of Tx Macros.

# Menus

Program menus offer many options for configuration and operation.  Explore them and test the resulting program actions.  The actions caused by most menu items should be self-explanatory.

File menu

Setup menu

View menu

Mode menu

Decode menu

Save menu

Help menu

# Keyboard Shortcuts

F1		Display online User's Guide in browser

Ctrl+F1	About WSJT-X

F2		Open the Setup | Configuration window

F3		Display keyboard shortcuts

F4		Clear Dx Call and Dx Grid entries

Alt+F4	Exit program

F5		Display special mouse commands

F6		Open next in directory

Shift+F6	Decode all remaining files in directory

F11		Move Rx frequency down 1 Hz

Ctrl+F11     	Move Rx and Tx frequencies down 1 Hz

F12		Move Rx frequency up 1 Hz

Ctrl+F12     	Move Rx and Tx frequencies up 1 Hz

Alt+D		Decode again at QSO frequency

Shift+D	Full decode (both windows)

Alt+E		Erase()

Ctrl+F		Edit the free text message box

Alt+G		Generate standard messages

Alt+H		Halt Tx()

Ctrl+L		Lookup callsign in database, generate standard messages

Alt M		Monitor()

Alt+N		Enable Tx()

Alt+Q		Log QSO()

Alt+S		Stop()

Alt+T		Tune

Alt+V		Save the most recently completed *.wav file

# Special Mouse Commands

| Mouse-click on | Action |
| --- | --- |
| Waterfall | Set Rx frequency Double-click to set Rx frequency and decode there Ctrl-click to set Rx and Tx frequencies Ctrl-double-click to set Rx and Tx frequencies and decode there |
| Decoded text | Double-click to copy second callsign to Dx Call and locator to Dx Grid; change Rx and Tx frequencies to decoded signal's frequency; generate standard messages. If first callsign is your own, Tx frequency is changed only if Ctrl is held down when double-clicking. |
| Erase button | Click to erase QSO window Double-click to erase QSO and Band Activity windows |
| Tx5 Entry | Right-click to select a macro message |

# Under the Hood

Since its origin in 2001, an important goal of the WSJT project has been education —bringing to Amateur Radio a better understanding of 21st century communication theory and applications.  I have needed (and acquired) plenty of this education, myself, while working on the project, and I think all of us contributing programming and other skills to the project feel that way.  We hope that users of our software will, also.  This brief section of the User’s Guide provides some technical details on how WSJT-X and the JT9 protocol work.

Transmitting:  Immediately before the start of a transmission, WSJT-X encodes a message and computes the sequence of tones to be sent.  As described in Appendix B, a JT9 sequence consists of 85 tone intervals or “symbols,” each at one of 9 pre-defined frequencies.  Tones are separated in frequency by the reciprocal of tone duration, so even for JT9-1, the fastest JT9 submode, the spacing is only 1.736 Hz — a difference scarcely perceptible to the human ear.  A JT9 signal sounds almost like an unmodulated carrier.

Sixteen transmitted symbols — those at positions 1, 2, 5, 10, 16, 23, 33, 35, 51, 52, 55, 60, 66, 73, 83, and 85 in the sequence — are always transmitted at the lowest (“tone 0”) frequency. Their frequency and their pseudo-random spacing in time is used to establish time and frequency synchronization between transmitter and receiver.  The remaining 69 intervals use tones 1 through 8, and they effectively send 69 × 3 = 207 information-carrying bits on their way into the ether.  WSJT-X computes the actual Tx audio waveform on-the-fly, using 16-bit integer samples and a 48000 Hz sample rate, maintaining phase continuity across symbol boundaries.  The digital samples are converted to an analog waveform in the sound card or equivalent USB interface.

Receiving:  The receiving side of WSJT-X acquires 16-bit integer samples from the sound card at a 12000 Hz rate.  The real input signal is filtered and converted to a complex (“analytic”) signal sampled at 1500 Hz.  Overlapping windowed spectra are computed for display and saved at intervals of half the symbol length.  They are used to produce the red curve (“JT9 Sync”) normally displayed below the waterfall in the Wide Graph window.  As shown in the screenshot on page 6, a JT9-1 signal appears in the red curve as an almost rectangular pulse about 16 Hz wide.  A slightly higher peak at the left edge shows the frequency of the synchronizing tone.  By convention, the nominal frequency of a JT9 signal is that of the sync tone.

Decoding:  At the end of a reception sequence, about 50 seconds into the UTC minute for submode JT9-1, the acquired complex samples are sent to the decoder.  For operator convenience the decoder goes through its full procedure twice: first covering a narrow range around the selected QSO Frequency,” and then over the full range between the blue markers at fmin and fmax.  Each pass can be described as a sequence of discrete steps.  For those who may wish to study the program’s source code, perhaps with an eye toward future improvements, the steps are presented here as blocks labeled with the names of functional procedures in the code.

sync9:	  Use sync symbols to find candidate JT9 signals

in specified frequency range

At the frequency of each plausible candidate:

downsam9:    Mix, filter and downsample to 16 complex samples/symbol

peakdt9:     Using sync symbols, time-align to start of JT9 symbol

sequence

afc9:	        Measure frequency offset and any possible drift

twkfreq:     Remove frequency offset and drift

symspec2:    Compute 8-bin spectra for 69 information-carrying

symbols, using the time- and frequency-aligned data;

transform to yield 206 single-bit soft symbols

interleave9: Remove single-bit symbol interleaving imposed at the

transmitter

decode9:     Retrieve a 72-bit user message using the sequential

"Fano" algorithm for convolutional codes

unpackmsg:   Unpack a human-readable message from the 72-bit

compressed format

It’s useful to know that when conditions necessary for decoding are marginal, the sequential algorithm can have exponentially long times to completion.  If the first step in the above sequence finds many seemingly worthy candidate signals, and if many of them turn out to be undecodable, the decoding loop can take a very long time.  For this reason the decode9 step in the above list is programmed to time out and report failure if it is taking too long.  The menu choice Decode | Fast / Normal / Deepest gives you three-step control of the timeout limit.

# Acknowledgments

Many users of WSJT, too numerous to mention here individually, have contributed suggestions and advice that have greatly aided the development of WSJT-X and its sister  programs.  Since 2005 the overall project (including WSJT, MAP65, WSPR, and WSPR-X) has been “open source”, all code being licensed under the GNU Public License (GPL).  For WSJT-X in particular, I wish to acknowledge contributions from AE4JY, PY2SDR, VK4BDJ, AC6SL, and AF5X.  Each has helped to bring the program’s design, code, and documentation to its present state.

# Appendix A: Installed and Generated Files

After installing WSJT-X as described in steps 1 and 2 on page 4, the following files will be present in the installation directory:

afmhot.dat			Data for AFMHot palette

blue.dat			Data for Blue palette

CALL3.TXT			Callsign database

hamlib-alinco.dll	Hamlib libraries

hamlib-amsat.dll		...

hamlib-flexradio.dll

hamlib-icom.dll

hamlib-jrc.dll

hamlib-kachina.dll

hamlib-kenwood.dll

hamlib-kit.dll

hamlib-tapr.dll

hamlib-tentec.dll

hamlib-winradio.dll

hamlib-yaesu.dll

jt9.exe			Executable for JT9 decoder

jt9code.exe           Test program to illustrate JT9 encoding

libfftw3f-3.dll		Optimized FFT library

libgcc_s_dw2-1.dll	gcc runtime

libhamlib-2.dll

libstdc++-6.dll		standard C function library

libusb0.dll		USB interface functions

mingwm10.dll		MinGW library

mouse_commands.txt	Special mouse commands

palir-02.dll		Linrad functions

PSKReporter.dll		Library for PSK reporter

QtCore4.dll		Qt libraries

QtGui4.dll			...

QtNetwork4.dll

QtSvg4.dll

qwt.dll			Qwt library

save				Directory for saved *.wav files

shortcuts.txt		Keyboard shortcuts

unins000.dat

unins000.exe		Executable for uninstalling WSJT-X

wsjt.ico			WSJT icon

wsjtx.exe			Executable for WSJT-X

You might be curious about additional files that appear in the WSJT-X installation directory after using the program for a while.  These include:

ALL.TXT		 Log of all received and transmitted messages

decoded.txt	 Decoded text from the most recent Rx interval

timer.out		 Diagnostic information for decoder optimization

wsjtx.ini		 Saved configuration parameters

wsjtx_log.adi	 ADIF log

wsjtx_status.txt	 Information sent to companion program JT-Alert

# Appendix B:  The JT9 Protocol

JT9 is a mode designed for making QSOs at HF, MF, and LF.  The mode uses essentially the same 72-bit structured messages as JT65.  Error control coding (ECC) uses a strong convolutional code with constraint length K=32, rate r=1/2, and a zero tail, leading to an encoded message length of (72+31) × 2 = 206 information-carrying bits.  Modulation is 9-FSK: 8 tones are used for data, one for synchronization.  Sixteen symbol intervals are used for synchronization, so a transmission requires a total of    206 / 3 + 16 = 85 (rounded up) channel symbols.  Symbol durations are approximately (TRperiod - 8) / 85, where TRperiod is the T/R sequence length in seconds.  Exact symbol lengths are chosen so that nsps, the number of samples per symbol (at 12000 samples per second) is a number with no prime factor greater than 7.  This choice makes for efficient FFTs.  Tone spacing of the 9-FSK modulation is df = 1 / tsym = 12000 / nsps, equal to the keying rate.  The total occupied bandwidth is 9 × df.  The generated signal has continuous phase and constant amplitude, and there are no key clicks.

Parameters of five JT9 submodes are summarized in the following table, along with approximate S/N thresholds measured by simulation on an AWGN channel.  Numbers following “JT9-” in the submode names specify TRperiod in minutes.

| Submode | nsps | Symbol Duration (s) | Tone Spacing (Hz) | Signal Bandwidth (Hz) | S/N Threshold* (dB) | QSO Time (minutes) |
| --- | --- | --- | --- | --- | --- | --- |
| JT9-1 | 6912 | 0.58 | 1.736 | 15.6 | -27 | 6 |
| JT9-2 | 15360 | 1.28 | 0.781 | 7.0 | -30 | 12 |
| JT9-5 | 40960 | 3.41 | 0.293 | 2.6 | -34 | 30 |
| JT9-10 | 82944 | 6.91 | 0.145 | 1.3 | -37 | 60 |
| JT9-30 | 252000 | 21.00 | 0.048 | 0.4 | -42 | 180 |

* Noise power measured in 2500 Hz bandwidth.

# Appendix C:  Source Code

WSJT-X is an open-source program released under the GNU General Public License. Source code is available from the public repository at http://developer.berlios.de/projects/wsjt/.  To compile the program you will need to install open source packages for Subversion, QtSDK, qwt, g++, g95 or gfortran, portaudio, fftw3, and hamlib.  For compiling in Windows I recommend installing the MinGW package.

The full source code for WSJT-X can be downloaded by using the command

svn co svn://svn.berlios.de/wsjt/branches/wsjtx

# Appendix D:  Troubleshooting

… to be added …